MCS7780 Driver Validation

Unload Issue

The Unload of the Driver has been tested multiples times with just plug/unplug. Also during the Data Transfer is in progress. It unloads with no problems.

To resolve Unload Issue steps to be followed till the New Fix has been released by Microsoft

- The Changes to be done in the irndis.c under \Wince500\Private\winceos\comm\ir\irlap\irndis.c at Line number 342. Just comment out #ifdef UNDER_CE and #endif at 344.

- Save the File irndis.c

- Go to Build OS, click on Open Release directory.

- Go to \Wince500\Private\winceos\comm\ir\irlap\ and run build -c

- Go to Build OS again, click on Build and Sysgen.

- During the build process, It will prompt you that some of the core files has been changed. Press OK for that.

- Copy the MosUsbIr.cab file to the WinCE 5.0 before connecting the Device using File Viewer. Remote File Viewer is found in the Tools sub Menu. The Serial Cable should be connected to make it run.

- Install the MosUsbIr.cab and connect the Device.

- Test for Load/Unload multiple times.

Rx/Tx Validation

| Speed | Rx(in KBytes per sec) |  | Tx(in KBytes per sec) |  |
| --- | --- | --- | --- | --- |
|  | Max | Min | Max | Min |
| 4 Mbps | 341 | 313 | 372 | 315 |
| 1152000 Mbps | 123 | 32 | 124 | 65 |
| 576000 Mbps | 36 | 18 | 66 | 65 |
| 115200 bps | 12 | 11 | 11.5 | 10.4 |
| 9600 bps | 0.83 | 0.71 | 1 | 0.93 |

MIR Speeds have been taken newly. Rest of the readings are old readings.