###### 5.1 Channel USB headset with Vibration

Vibration headphones with microphone

User manual

Introduction

Thank you for buying USB 5.1 surround headset! This manual will help you install and use it properly.

The headset is a 5.1 channel, high sound quality headset with microphone and USB interface with the following main features:

- 5.1 channel

- High sound quality and vibration

- Adjustable microphone

- Volume control

- Plug and play

- Soft leather ear pads

Package contents

Please check the following package contents of your USB 5.1 surround headset:

- 1 x Headset with microphone

- 1 x CD with software

Software requirements

- Windows 98SE, ME, 2000, or XP

- Direct 3D

Hardware requirements

- USB 1.1 and 2.0

- Intel Pentium III, AMD K6 500 CPU or above

- At least 64 MB RAM (Win98SE) or 128MB RAM (WinME, Win2000 and WinXP)

- At least 50 MB hard disk space

Important

Before installing Xear 3D audio, plug in the USB 5.1 Headset first. Always use the same USB PC connector for the USB 5.1 Headset to automatically start the Xear 3D software when the headset is plugged in.

Software installation

- Turn on the PC system and connect the USB 5.1 Headset to a USB connector.

- Insert the installation CD into your CD drive. If auto run is activated, the setup program will start up automatically. Otherwise, start the setup program “setup.exe” from the CD drive by hand.

If this message appears, please Plug-In the USB 5.1 first, wait a few seconds until the headset is recognized, and continue the installation by clicking “OK”.

- Click the “Next” button to continue the installation.

Select “Install USB Audio Driver” for installing the Xear 3D driver software. Selecting “Install 3D Audio Program” installs two simple, non-violent 3D sound demo applications, a helicopter game and a rock shooting game. They can be started from the “Start Menu” – “USB Audio”. Click “Next” to continue

- Set up will install the application into the default path and folder.

- If you run Windows XP, you may see the following dialog box, click “Continue Anyway” to continue the installation.

- The copy process may take a little while, wait until it finishes.

- Reboot to complete the software installation, otherwise the driver will not be initialized.

- After the reboot you will see a new blue icon in the Systray. Click on it to open the Xear 3D Sound driver application.

Note: For the driver to recognize the headset, it has to be plugged into the USB port from where it was originally installed. After unplugging the headset, make sure to connect it back into the same USB port, otherwise the Xear 3D sound icon will not show in the Systray. Should the Xear 3D icon ever be missing, the USB connector is most likely in the wrong connector.

User Interface and Functions

Main Settings

- Analog Output: Click on this button for turning on analog output.

- Analog Output Channels: Select Earphones.

- Digital Output: Turns on SPDIF digital data output.

- Digital Output Mode: Dolby RTCE or PCM (depending on configuration).

- Available Audio Jacks: Does not apply to AP5.1

- DSP Mode: Pull down to select Xear 3D DSP Mode or none to turn off DSP mode.

- Virtual Speaker Shifter: Turns on/off Speaker Shifter controls (see details below).

- Digital Volume Control: Allows adjusting each channel’s volume (see details below).

- Speaker/Channel Display: Shows analog output channels and allows direct speaker controls and shifting (see details below).

- Auto Sound Test: Performs an auto test for each channel.

- Stop Auto Sound Test: Stops auto testing.

Virtual Speaker Shifter

|  | 1: Total Volume Control Up |
| --- | --- |
|  | 2: Total Volume Control Down |
|  | 3: Rotate Speakers Clockwise |
|  | 4: Rotate Speakers Counter Clockwise |
|  | 5: Rotate Speakers Manually |
|  | 6: Shift Speakers Manually |
|  | 7: Reset to Standard Settings |
|  | 8: Drag Area |

Digital Volume Control (Speaker Shifter turned off)

|  | 1: Reset Volume |
| --- | --- |
|  | 2: Left Channel Control |
|  | 3: Right Channel Control |

Mixer

1: Master Balance 		   9: SW Synth On/Off

2: Master Volume			  10: CD Balance

3: Master Volume On/Off 	  11: CD Volume Control

4: Wave Balance  		  12: CD On/Off

5: Wave Volume		        13: Microphone Balance

6: Wave On/Off         	  14: Microphone Volume

7: SW Synth Balance		  15: Microphone On/Off

8: SW Synth Volume		  16: Microphone Advanced

Settings

1

2

3

1: Microphone Record Balance

2: Microphone Record Volume

3: Microphone Record Volume On/Off

Effects

Environment Settings

1: Bathroom

2: Opera

3: Underwater

4: Music Pub

5: More options: 27 additional modes

Environment Size

6: Large

7: Middle

8: Small

Equalizer

9: Ten Band Equalizer Controls		24: User defined name input

10 - 21: Pre-Set Modes				25: Add new user defined mode

22: Equalizer User Defined On/Off		26: Delete user defined mode

23: Select user define modes

Karaoke

- Karaoke On/Off 
Click this button to control all Karaoke functions.

- Reset all Karaoke settings to their default values.

- Enable microphone echo. A pop up box will appear, click the “OK” button to select recording source and enable microphone.

- Echo adjust

- Selected echo function.

- Enable Key Shifting Function.

- Key Shifting Control for adjusting the background music.

- Check box to enable the Vocal Cancellation function.

- Vocal Cancellation control.

Cable - Audio Controller description

1: Volume Down

2: Volume Up

3: Microphone Mute

4: LED: Headphone Indicator (flashes when active)

5. Vibration Switch(only for headset with vibration function)

This menu shows Xear 3D USB audio product and system information, it can be useful for trouble shooting or when you contact technical support.

Troubleshooting

| Problem description | Proposed solution |
| --- | --- |
| Why is there no sound during the CD playback? | The CD audio is probably set to analog instead of digital audio. The USB audio signal is digital, therefore the input signal has to be digital as well. Open the “Device Manager”, select “DVD/CD ROM Drives” and open it. Then open the “Properties” page and enable “Digital CD Audio”. |
| Why is it that Win 98SE and Win ME sometimes cannot auto detect the USB audio device? | Win 98SE and Win ME have different driver architecture than Win 2000 and Win XP. When users plug in/out the USB audio device too fast, Win 98SE and ME may not detect the device change. In this case you will have to switch the audio device to “USB audio device” by yourself. Open the “Control Panel”, select “Sounds and Audio Devices”, choose “Audio” and set “Default device” to “USB Device” or “Gamtec USB Device”. |
| Can I unplug the USB audio device while playing back audio files with a software application such as Windows Media player, Winamp, PowerDVD, WinDVD? | No, stop playback first! Otherwise your Computer may become unstable. |

Specification:

USB Headset With 5.1 Channel Sound and Vibration Speakers Specification. Driver Unit:; 50 mm diameter. Impedance:; 32Ω±3Ω. Frequency:; 20Hz ~ 20KHz. Sensitivity:; 110 ± 3dB. Max. Input:; 150mW...

Microphone Specification. Directivity:. Omni-directionalImpedance:; 2.2KΩ at 1K Hz. Frequency:; 20Hz ~ 16KHz. Sensitivity:; -58 ± 3dB...